CONNECTION INFORMATION

The transmitter will transmit under 2 conditions

- When the power is applied.

- With the power permanently applied, simply open circuit the N/C to negative loop.

In both circumstances the transmitter will show the transmission by the transmission LED.

The transmissions will time out after a few seconds.

Therefore to connect the HPATX to a N/O contact simply insert the N/O contact in series with the supply or connect across the siren if this is the signal to be sent.

For the N/C contact, simply connect the mag contact or passive IR N/C loop to the N/C terminal of the HPATX and negative. With the supply connected, as soon as the loop is opened, there will be a transmission. This will be indicated by the transmit LED.

NOTES ON INSTALLATION

When fitting the HPATX, keep the aerial away from metal objects such as metal poles or steel sheeting. These will absorb a portion of the signal and thereby reducing the range.

4

High Powered Alarm TX

Item code

HPATX

INDEX

Specifications				P2

Connection layout information	P3

Connection information			P4

SPECIFICATIONS

Input voltage				15 V DC 11V AC

Input current				30mA

Frequency of Operation		403, 55 MHz or

433, 92 MHz

Others available on

request

Output power	6mw ERP

Coding format					9 Way trinary

Maximum number of codes			19683

Type of contacts					N/O and N/C

2

CONNECTION LAYOUT INFORMATION

Negative

Positive

N/C Trigger

3